APPENDIX – I I

SOURCE CODE OF THE SUBSTATION MONITORING SYSTEM

SOFTWARE  DESCRIPTION

SOFTWARE CODES

Option Explicit

Dim stop_flag, Timer_flag As Boolean

Dim Store_flag As Boolean

Dim tim, storetime As Variant

Private Sub about_Click()

frmAbout.Show

End Sub

Private Sub analyze_Click()

SelectParam.Show

End Sub

Private Sub com1_Click()

com1.Checked = True

com2.Checked = False

If MSComm1.PortOpen = True Then

MSComm1.PortOpen = False

MSComm1.commport = 3

If MSComm1.PortOpen = False Then

MSComm1.PortOpen = True

End If

End If

End Sub

Private Sub com2_Click()

com2.Checked = True

com1.Checked = False

If MSComm1.PortOpen = True Then

MSComm1.PortOpen = False

MSComm1.commport = 2

If MSComm1.PortOpen = False Then

MSComm1.PortOpen = True

End If

End If

End Sub

Private Sub Exit_Click()

Unload Me

End Sub

Private Sub diag_Click()

SendDiag

End Sub

Private Sub eepdata_Click()

ReadEEPROM

End Sub

Private Sub Form_Activate()

readnode_Click

End Sub

Private Sub Form_Load()

Misc.Enabled = False

MSComm1.commport = 1

If MSComm1.PortOpen = False Then

MSComm1.PortOpen = True

End If

NodeID = 1

storetime = 1

tim = 0

End Sub

Private Sub min1_Click()

storetime = 1

End Sub

Private Sub min5_Click()

storetime = 5

End Sub

Private Sub min15_Click()

storetime = 15

End Sub

Private Sub min30_Click()

storetime = 30

End Sub

Private Sub min60_Click()

storetime = 60

End Sub

Private Sub readnode_Click()

Timer1.Enabled = True

stop_flag = False

Timer_flag = True

Misc.Enabled = False

Do

DoEvents

If Timer_flag = True Then

Text2.Text = ""

ComMessage = ""

Save.Visible = False

Timer_flag = False

If SendDiag = True Then

Text2.Text = ComMessage

If ReadRegister(0, 25) = True Then

If Store_flag = True Then

Save.Visible = True

Text2.Text = "Storing DATA into the Data base..."

Store_flag = False

Store_Data (NodeID)

End If

End If

Text2.Text = ComMessage

End If

Calculate

End If

Loop Until stop_flag = True

End Sub

Private Sub select_Click()

NodeID = InputBox("Enter the Node ID (1..255):", "STATION ID", NodeID)

End Sub

Private Sub Stop_Click()

stop_flag = True

Misc.Enabled = True

End Sub

Private Sub Timer1_Timer()

Timer_flag = True

End Sub

Private Sub Form_Unload(Cancel As Integer)

Unload Me

End

End Sub

Private Sub Timer2_Timer()

tim = tim + 1

If tim = storetime Then

Store_flag = True

tim = 0

End If

End Sub

/* CODE USED IN SELECTPARAM.FRM*/

Option Explicit

Private Sub CancelButton_Click()

SelectParam.Hide

End Sub

Private Sub Form_Load()

Dim myname

myname = Dir$(app.Path & "\*.dbf", vbHidden)

Do While myname <> ""   ' Start the loop.

cmbDate.AddItem Mid$(myname, 1, InStr(myname, ".") - 1)

myname = Dir   ' Get next entry.

Loop

cmbDate.Text = cmbDate.List(0)

End Sub

Private Sub OKButton_Click()

GraphDate = cmbDate.Text

Param = cmbParam.Text

StartTime = StartT

EndTime = EndT

TREND.Show

End Sub

/* CODES USED TRENDGRAPGH.FRM*/

Option Explicit

Dim rs1 As ADODB.Recordset

Dim cn As New ADODB.Connection

Private Sub Form_Activate()

Dim rCount As Integer

TREND.MousePointer = vbHourglass

If Param = "Voltage" Then

Set rs1 = cn.Execute("SELECT Times, Volt1, Volt2, Volt3 FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

ElseIf Param = "Current" Then

Set rs1 = cn.Execute("SELECT Times, Curr1, Curr2, Curr3 FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

ElseIf Param = "Power Factor" Then

Set rs1 = cn.Execute("SELECT Times, Pf1, Pf2, Pf3 FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

ElseIf Param = "kW" Then

Set rs1 = cn.Execute("SELECT Times, Kw1, Kw2, Kw3 FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

ElseIf Param = "kVA" Then

Set rs1 = cn.Execute("SELECT Times, Kva1, Kva2, Kva3 FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

ElseIf Param = "kVAr" Then

Set rs1 = cn.Execute("SELECT Times, Kvar1, Kvar2, Kvar3 FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

ElseIf Param = "kWH" Then

Set rs1 = cn.Execute("SELECT Times, Kwh FROM " & GraphDate & " where nodeid = 1 and Times between '" & StartTime & "' and '" & EndTime & "' order by Times")

End If

If rs1.EOF = False Then

rs1.MoveFirst

Else

MsgBox "No Record for the spefied Time"

End If

rCount = 0

If Param <> "kWH" Then

MSChart1.ColumnCount = 3

Else

MSChart1.ColumnCount = 1

End If

MSChart1.RowLabel = "Time"

MSChart1.ColumnLabel = Param

MSChart1.TitleText = "Off-Line Graph: " & Param

Do While rs1.EOF = False

rCount = rCount + 1

MSChart1.RowCount = rCount

MSChart1.Row = rCount

MSChart1.RowLabel = rs1.Fields(0) 'TIME

MSChart1.Column = 1

MSChart1.Data = Format(Abs(rs1.Fields(1)), "0.00")

If Param <> "kWH" Then

MSChart1.Column = 2

MSChart1.Data = Format(Abs(rs1.Fields(2)), "0.00")

MSChart1.Column = 3

MSChart1.Data = Format((Abs(rs1.Fields(3))), "0.00")

End If

rs1.MoveNext

If rs1.EOF = True Then

Me.MousePointer = vbDefault

Exit Do

End If

Loop

rs1.Close

TREND.MousePointer = vbDefault

TREND.MousePointer = vbDefault

End Sub

Private Sub Form_Load()

cn.CursorLocation = adUseClient

cn.ConnectionString = "driver=Microsoft dBase Driver (*.dbf);DBQ=" & app.Path

cn.Open

End Sub

Private Sub Form_Terminate()

Unload Me

End Sub

Private Sub Form_Unload(Cancel As Integer)

cn.Close

Unload Me

End Sub

Private Sub PrintGraph_Click()

Me.PrintForm

Exit Sub

End Sub

Private Sub MSChart1_PointSelected(Series As Integer, DataPoint As Integer, MouseFlags As Integer, Cancel As Integer)

Dim s As String

MSChart1.Row = DataPoint

MSChart1.Column = Series

If Param = "Voltage" Then

MSChart1.ToolTipText = "Time =" & MSChart1.RowLabel & ", Data = " & MSChart1.Data & "(Volts)"

ElseIf Param = "Current" Then

MSChart1.ToolTipText = "Time =" & MSChart1.RowLabel & ", Data = " & MSChart1.Data & "(Amps)"

ElseIf Param = "Power Factor" Then

MSChart1.ToolTipText = "Time =" & MSChart1.RowLabel & ", Data = " & MSChart1.Data

ElseIf Param = "kW" Then

MSChart1.ToolTipText = "Time =" & MSChart1.RowLabel & ", Data = " & MSChart1.Data & "(kW)"

ElseIf Param = "kVA" Then

MSChart1.ToolTipText = "Time =" & MSChart1.RowLabel & ", Data = " & MSChart1.Data & "(kVA)"

ElseIf Param = "kVAr" Then

MSChart1.ToolTipText = "Time =" & MSChart1.RowLabel & ", Data = " & MSChart1.Data & "(kVAr)"

End If

End Sub

/* CODE used in CALC.BAS */

Dim Status As Byte

Dim vlt(3), cur(3), pfa(3), kw(3), kva(3), kvar(3) As Single

Dim avolt, acurr, apf, akw, akva, akvar, freq As Single

Dim ncurr As Double

Dim kwh, kvarh As Double

Dim CTRATIO As Integer

Public GraphDate, Param, StartTime, EndTime As String

Public Sub Calculate()

Dim lop As Byte

Dim realpart, imagpart As Double

Dim cosx(3), sinx(2) As Single

On Error Resume Next

avolt = 0

acurr = 0

apf = 0

akw = 0

akva = 0

akvar = 0

For lop = 0 To 2

vlt(lop) = Dat_Buf(lop * 3 + 3)

cur(lop) = Dat_Buf(lop * 3 + 4)

pfa(lop) = Dat_Buf(lop * 3 + 5)

kw(lop) = vlt(lop) * cur(lop) * Abs(pfa(lop))

kva(lop) = vlt(lop) * cur(lop)

kvar(lop) = Sgn(pfa(lop)) * Sqr(kva(lop) * kva(lop) - kw(lop) * kw(lop))

avolt = avolt + 1.732 * vlt(lop)

acurr = acurr + cur(lop)

apf = apf + pfa(lop)

akw = akw + kw(lop)

akva = akva + kva(lop)

akvar = akvar + kvar(lop)

Main.volt(lop).Text = Format(vlt(lop), "0.00 V")

Main.curr(lop).Text = Format(cur(lop), "0.00 A")

Main.pf(lop).Text = Format(pfa(lop), "0.00")

Main.ap(lop).Text = Format(kw(lop) / 1000, "0.000 kW")

Main.rp(lop).Text = Format(kvar(lop) / 1000, "0.000 kVAr")

Main.app(lop).Text = Format(kva(lop) / 1000, "0.000 kVA")

cosx(lop) = kw(lop) / kva(lop)

sinx(lop) = kvar(lop) / kva(lop)

Next lop

avolt = avolt / 3

acurr = acurr / 3

apf = apf / 3

Main.Text1(0).Text = Format(avolt, "0.00 V")

Main.Text1(1).Text = Format(acurr, "0.00 A")

Main.Text1(2).Text = Format(apf, "0.00")

Main.Text1(3).Text = Format(akw / 1000, "0.00 kW")

Main.Text1(4).Text = Format(akva / 1000, "0.00 kVA")

Main.Text1(5).Text = Format(akvar / 1000, "0.00 kVAr")

kwh = Abs(Dat_Buf(0) / 1000)

kvarh = Abs(Dat_Buf(1) / 1000)

freq = Dat_Buf(2)

Status = Dat_Buf(12)

Main.ae.Text = Str(Format(kwh, "0.000")) + " kWh"

Main.re.Text = Str(Format(kvarh, "0.000")) + " kVArh"

Main.freq.Text = Format(freq, "0.00 Hz")

' Neutral Current Calculation

realpart = cur(0) * cosx(0) + cur(1) * ((-0.5) * cosx(1) - 0.866 * sinx(1)) + cur(2) * ((-0.5) * cosx(2) + 0.866 * sinx(2))

imagpart = cur(0) * sinx(0) + cur(1) * (0.866 * cosx(1) - 0.5 * sinx(1)) + cur(2) * ((-0.866) * cosx(2) - 0.5 * sinx(2))

ncurr = Sqr(realpart * realpart + imagpart * imagpart)

Main.curr(3) = Format(ncurr, "0.00")

' Display status

Main.Status(0).FillColor = vbGreen

Main.Status(1).FillColor = vbGreen

Main.Status(2).FillColor = vbGreen

Main.Status(3).FillColor = vbGreen

If Status And &H40 Then     ' Phase Sequence Error

Main.Status(3).FillColor = vbRed

End If

If Status And &H20 Then     ' Missing Phase Error

If Status And &H10 Then Main.Status(2).FillColor = vbRed    ' Phase 3 Missing

If Status And &H8 Then Main.Status(1).FillColor = vbRed     ' Phase 2 Missing

If Status And &H4 Then Main.Status(0).FillColor = vbRed     ' Phase 1 Missing

End If

End Sub

Public Sub Store_Data(id As Integer)

Dim fname As String

Set cn = New ADODB.Connection

cn.CursorLocation = adUseClient

cn.ConnectionString = "driver=Microsoft dBase Driver (*.dbf);DBQ=" & app.Path

cn.Open

fname = Format(Date, "ddmmyy")

If Dir$(app.Path & "\" & fname & ".dbf") = "" Then

cn.Execute " create table " & fname & "(NodeID byte ,times text(10) ,volt1 double ,volt2 double ,volt3 double ,curr1 double ,curr2 double ,curr3 double ,pf1 double ,pf2 double ,pf3 double ,kw1 double  ,kw2 double  ,kw3 double ,kvar1 double ,kvar2 double ,kvar3 double ,kva1 double ,kva2 double ,kva3 double ,kwh double ,kvarh double ,freq double ,ncurr double)"

End If

cn.Execute ("insert into " & fname & " values (" & id & ",'" & Format(Time, "hh:mm") & "'," & vlt(0) & "," & vlt(1) & "," & vlt(2) & "," & cur(0) & "," & cur(1) & "," & cur(2) & "," & pfa(0) & "," & pfa(1) & "," & pfa(2) & "," & kw(0) & "," & kw(1) & "," & kw(2) & "," & kvar(0) & "," & kvar(1) & "," & kvar(2) & "," & kva(0) & "," & kva(1) & "," & kva(2) & "," & kwh & "," & kvarh & "," & freq & "," & ncurr & ")")

cn.Close

End Sub

/* CODES USED IN MODBUS.BAS*/

Option Explicit

Public cn As New ADODB.Connection

Public rs As New ADODB.Recordset

Public Declare Function LdExp Lib "dllbasic" (ByVal x As Single, ByVal y As Integer) As Single

Public Declare Function Rtl Lib "dllbasic" (ByVal value As Long, ByVal cnt As Byte) As Long

Public Declare Function Rtr Lib "dllbasic" (ByVal value As Long, ByVal cnt As Byte) As Long

Public NodeID As Variant

Public ComMessage As String

Public Dat_Buf(12) As Single

Dim PackData, pack, id As Variant

Dim chksum As Variant

Dim rec_buff(100) As Variant

Dim err_flag As Boolean

Dim strData, chkstr As String

Dim exception_flag As Boolean

Public Sub SendPack()

Main.MSComm1.DTREnable = False

Main.MSComm1.Output = strData

Main.MSComm1.DTREnable = True

End Sub

Public Sub FramePacket()

Dim i As Integer

For i = 0 To 12

Dat_Buf(i) = 0

Next i

strData = Main.MSComm1.Input

strData = ""

SendPack

SendPack

SendPack

SendPack

For i = 0 To 5

strData = strData & Chr(PackData(i))

Next i

chkstr = CalCRC16

strData = strData & chkstr

SendPack

strData = ""

SendPack

SendPack

SendPack

SendPack

End Sub

Public Function SendDiag() As Boolean

Dim i As Variant

PackData = Array(NodeID, 8, 0, 0, &HA5, &H37, 0, 0)

FramePacket

Receive (8)

Commcheck

Main.Text3.Text = ""

If err_flag = False And exception_flag = False Then

For i = 0 To 5

Main.Text3.Text = Main.Text3.Text & Asc(Mid(strData, i + 1, 1)) & "   "

Next i

SendDiag = True

Else

SendDiag = False

End If

End Function

Public Function ReadRegister(addr As Integer, Regcnt As Integer) As Boolean

Dim msb, lsb As Byte

Dim i, j, k As Variant

Dim bytecnt As Integer

Dim tmp(4) As Variant

msb = CByte(addr / 256)

lsb = CByte(addr Mod 256)

PackData = Array(NodeID, 3, msb, lsb, 0, Regcnt, 0, 0)

FramePacket

bytecnt = Regcnt * 2

Receive (bytecnt + 5)

Commcheck

If err_flag = False And exception_flag = False Then

k = 0

For i = 4 To bytecnt - 2 Step 4

For j = 0 To 3

tmp(j) = Asc(Mid(strData, i + j, 1))

Next j

If tmp(0) > &H7F Then

Dat_Buf(k) = -1

tmp(0) = tmp(0) And &H7F

Else

Dat_Buf(k) = 1

End If

Dat_Buf(k) = Dat_Buf(k) * Long_Float(tmp(0) * 2 ^ 24 + tmp(1) * 2 ^ 16 + tmp(2) * 2 ^ 8 + tmp(3))

k = k + 1

Next i

Dat_Buf(k) = Asc(Mid(strData, bytecnt + 2, 1))

ReadRegister = True

Else

ReadRegister = False

End If

End Function

Public Function ReadEEPROM() As Boolean

Dim i, j, k As Variant

Dim bytecnt As Integer

Dim tmp(4) As Variant

PackData = Array(NodeID, 4, 0, 0, 0, 45, 0, 0)

FramePacket

Receive (50)

Commcheck

Main.Text3.Text = ""

If err_flag = False And exception_flag = False Then

ReadEEPROM = True

k = 0

For i = 4 To 48

Main.Text3.Text = Main.Text3.Text & Hex(Asc(Mid(strData, i, 1))) & "  "

Next i

Else

ReadEEPROM = False

End If

End Function

Public Sub Receive(cnt As Variant)

Dim times As Long

Dim ncount As Variant

strData = ""

times = 50000

Do

DoEvents

ncount = Main.MSComm1.InBufferCount

times = times - 1

Loop Until ((ncount = cnt) Or (ncount = 5) Or (times = 0))

If (ncount = 5 And ncount <> cnt) Then exception_flag = True

err_flag = True

If times = 0 Then Exit Sub

Main.MSComm1.InputLen = ncount

strData = Main.MSComm1.Input

chkstr = Right(strData, 2)

strData = Mid$(strData, 1, Len(strData) - 2)

If chkstr = CalCRC16 Then err_flag = False

End Sub

Public Function CalCRC16() As String

Dim lCrc, lGenerator As Long

Dim i, j As Integer

Dim byTemp As Byte

Dim bBit As Boolean

lCrc = 65535

lGenerator = 40961

For i = 1 To Len(strData)

byTemp = Asc(Mid(strData, i, 1))

lCrc = lCrc Xor byTemp

For j = 1 To 8

bBit = lCrc And 1

lCrc = lCrc \ 2

If bBit = True Then lCrc = lCrc Xor lGenerator

Next j

Next i

CalCRC16 = Chr(lCrc Mod 256) & Chr(lCrc \ 256)

End Function

Public Sub Commcheck()

Dim response As Byte

If err_flag = False Then

ComMessage = "COMMUNICATION IS OK"

ElseIf err_flag = True Then

ComMessage = "COMMUNICATION ERROR"

Exception_Record NodeID, 0, ComMessage

End If

If exception_flag = True Then

If strData <> "" Then

response = Asc(Mid(strData, 3, 1))

ComMessage = "!Exception Error!"

Select Case (response)

Case 1

ComMessage = "Exception Error: ILLEGAL FUNCTION"

Case 2

ComMessage = "Exception Error: ILLEGAL DATA ADDRESS"

Case 3

ComMessage = "Exception Error: ILLEGAL DATA VALUE"

Case 4

ComMessage = "Exception Error: SLAVE DEVICE FAILURE"

Case 5

ComMessage = "Exception Error: ACKNOWLEDGE"

Case 6

ComMessage = "Exception Error: SLAVE DEVICE BUSY"

Case 7

ComMessage = "Exception Error: NEGATIVE ACKNOWLEDGE"

Case 8

ComMessage = "Exception Error: MEMORY PARITY ERROR"

End Select

Exception_Record NodeID, response, ComMessage

End If

End If

End Sub

Public Function Long_Float(value As Long) As Single

Dim lMant, iExpo As Long

Dim i As Integer

Dim j As Single

lMant = value And &H7FFFFF

iExpo = (Rtl(value, 23) And &HFF) - 127

j = 0

For i = 1 To 23

If lMant And &H400000 Then

j = j + 2 ^ (-i)

End If

lMant = Rtr(lMant, 1)

Next i

j = (j + 1) / 2

If value > &H7FFFFFFF Then j = j * (-1)

Long_Float = LdExp(j, iExpo + 1)

End Function

Public Sub Exception_Record(id As Variant, excepcode As Byte, message As String)

Dim fname As String

Set cn = New ADODB.Connection

cn.CursorLocation = adUseClient

cn.ConnectionString = "driver=Microsoft dBase Driver (*.dbf);DBQ=" & app.Path

cn.Open

fname = "ComError"

If Dir$(app.Path & "\" & fname & ".dbf") = "" Then

cn.Execute " create table " & fname & "(eDate text(10) ,eTime text(10) ,eID byte ,ExcepCode byte ,ExcepDesc text(20))"

End If

cn.Execute ("insert into " & fname & " values ('" & Format(Date, "dd/mm/yy") & "','" & Format(Time, "hh:mm:ss") & "', " & id & "," & excepcode & ",'" & message & "')")

cn.Close

End Sub

- 0 0 0  -

APPENDIX – III

USER  INTERFACE  OF  THE  SUBSTATION MONITORING SYSTEM

Software Menus and its Description:

Main Screens:

Figure shows the Main Screen of the EMS software. This screen displays the information about the phase-wise measure voltage, current, power factor, active, reactive, apparent powers, energies, frequency and the status information about the load or feeder. The status gives the information about the phase sequence error or missing phase etc. The 3-phase equivalent information like Line-Line voltage, average current, average power factor, total kW, kVA, kVAr etc also displayed.

Screen-1

Menu Description:

The menu bar consists of five items. They are

- MAIN Menu

- MISC. Menu

- ANALYZE Menu

- ABOUT EMS menu

- EXIT menu

MAIN Menu:

This menu consists of two items; namely

- SELECT NODE ID

- SELECT INTERVAL

SELECT NODE ID:

Figure shows the snapshot of the Screen when the user selects the SELECT NODE ID menu. One can select the STATION ID from 0 -255.

Screen-2

SELECT INTERVAL:

The user can select the Data Storage interval of 1, 5, 15, 30, 60 minutes for storing the information in the database.

Screen - 3

MISC. Menu:

This menu is basically hidden for the general user and only the authorized person can use this command. This menu consists of the following items;

- COM PORT

- DIAGNOSE

- READ EEPROM DATA

- READ REGISTER.

Screen - 4

COM PORT:

This sub-menu is used to select the Communication port to be used for RS-485 serial communication. The options regarding the available COM ports will be automatically displayed. One can choose the connected COM port.

Screen - 5

DIAGNOSE, READ EEPROM DATA, READ REGISTER Menu:

These commands are used to check the communication in step-by-step mode for network problems.

ANALYZE Menu:

This menu is used to analyze the data and to generate the reports. When this menu is clicked, then it displays the dialog box, which is used to select the DATE, Parameter, Start & Stop time of the interest.

Screen -6

Once the Date, parameter, start time and end time are selected one can click the SHOW GRAPH command. This will show the graph for the selected parameter with the specified time frame. One can go for hard copy of the same by clicking the Printer option. Figure shows one such off-line graph for the Voltage parameter.

Screen -7

ABOUT EMS:

This shows the EMS description and the Company Name etc.

Screen - 8

EXIT Menu:

This menu is used to close the EMS application.

Screen - 9         Instantaneous Current Report

Screen -10       Energy  Test Report

- 0 0 0 -