CERTIFICATE

This is to certify that the project entitled “Effect of non-uniformity in isolator stiffness on torsional coupling of superstructure” is a record of the bonafide dissertation work carried out by Me, Meenakshi Joshi, Roll No. 07/str/06, student of Master of Engineering in Civil (structural) Engineering from Delhi College of Engineering, Delhi, during the session 2007-08 towards the partial fulfillment of the requirements for award of the degree of Master of Engineering in Civil (Structural) Engineering.

Meenakshi Joshi

College Roll. No. (07/str/06)

University Roll. No. 10306

M.E. IV Sem. (Structural Engg.)

Delhi College of Engineering.

DELHI - 110042

The above stated statement by Meenakshi Joshi, is correct to the best of our knowledge.

Date:

#### Prof. P. R. BOSE                               Dr. A. K. GUPTA (Sr.)

Prof. Delhi College of Engineering &                        Department of Civil &

Principal, C.B.P. Govt. Engg. College                       Environmental Engineering,

Jaffarpur , Delhi                                                          Delhi College of Engg., Delhi.

ACKNOWLEDGEMENT

This report is an outcome of persistent effort and a great deal of dedication and it has drawn intellectual and moral support from various people within & outside the institution.

i feel privileged in expressing my earnest obligations, deep sense of gratitude, appreciation & honor to both of my guides Prof. P.R. BOSE, Prof. Delhi College of Engineering & Principal, C.B.P. Govt.  Engineering College, Jaffarpur, Delhi , & Dr. A. K. GUPTA (Sr.), Department of Civil Engineering, Delhi College Engineering, for their constant guidance and unflinching encouragement throughout the development of this project. Their benevolent guidance apt suggestions, unstinted help & constructive criticism have inspired me in successful completion of this dissertation work.

It is a matter of great pride for me to have worked under them, which in itself was a source of inspiration for me to complete the project with great enthusiasm, energy and determination.

I humbly extend my words of gratitude to Prof. C.K. DUTTA, Head of the department & other faculty members, laboratory staff and administration of Civil & Environmental Engineering Department for free exchange of ideas & discussions which proved quite helpful &  for providing me the valuable help and time whenever it was required.

I would like to extend my thanks to Dr. Vasant Matsagar (Michigan University) and Dr. Rajeev Sharma (CSI, India) for their kind help & constant online guidance which they provided me whenever needed. I am really grateful to Dr. Rajeev Sharma for helping me out for easy working with SAP2000 Version 11 (Advanced).

I am obliged to Prof. R.S. Jangid, Professor, Department Of Civil Engineering, IIT Bombay for his kind guidance & technical support during my project. I am also thankful to Professor Durgesh Rai of Department of Civil Engineering IIT Kanpur, Prof. C.V.R. Murthy of IIT Hyderabad & professor Yogendra singh, Department of  Earthquake Engineering IIT Roorkee for sparing their precious time for me to help me out with my doubts.

And I would also like to thank Er. Sushil Bhargava & Er. Brijesh (Public Works Department, Delhi) for their support at the time of site visit.  And I would like to convey my words of gratitude to Er. A. Sen (Design Consultant) for  sparing his much precious time to have  technical discussions which proved quite useful, at the initial stage of  the project.

I am indebted to my family (Parents, Siblings & Grand ma) and my friends (Dolly, Sakshi, Yogi, Urmi, Abhishek Singh, Sonal, Amit  &  all ) for helping me realize my goals & for their constant encouragement, love & support. Without their blessings, this work would not have been possible.

Lastly, I thank Almighty GOD for his countless blessings.

Date:  								Meenakshi joshi

(07/str/06)

CONTENTS

- CERTIFICATE………………………………………………………….. i

ACKNOWLEDGEMENT…………………………………………….....ii

CONTENTS…………………………………………………………......iv

LIST OF FIGURES……………………………………………………..vii

LIST OF TABLES…………………………………………………….....x

LIST OF SYMBOLS & ABBREVIATIONS …………………………..xi

ABSTRACT……………………………………………………….........xv

1. INTRODUCTION & LITERATURE REVIEW………………......1

1.1 Background………………………………………………………….1

1.1.1Seismic retrofitting…..……………………………………………...2

1.1.2Aseismic retrofitting…...……………………………………………5

1.2 Base isolation………………………………………………………...6

1.2.1Suitability of base isolation…………………………………………8

1.3Types of base isolation systems……………………………………..9

1.3.1 Elastomeric bearings………………………………………………10

1.3.1.1Laminated rubber bearings………………………………………10

1.3.1.2 New Zealand (NZ) system………………………………………11

1.3.2Sliding systems…………………………………………………….12

1.3.2.1Pure friction system (PF)………………………………………...12

1.3.2.2Resilient friction base isolators (R-F)……………………………13

1.3.2.3Friction pendulum system (FPS)………………………………...14

1.3.2.4Electricide De France (EDF)…...………………………………..15

1.3.2.5Slinding resilient Friction bearing (SR-F)……………………….15

1.4Principlof base isolation…………………………………………...16

1.5 Applications of base isolation technique worldwide for seismic retrofitting…………………………………………………………….18

1.6 Base Isolation: Execution of Retrofitting work………………….19

1.6.1The installation sequence for a typical LRB into a RCC column…19

1.6.2 Jacking & Re-support of existing column for steel structure……..25

2.OBJECTIVE OF PRESENT WORK……………………...……….26

3.BASE ISOLATION …………...…………………………………….27

3.1Base isolator………………………………………………………...27

3.2Fixed base structure………………………………………………..29

3.3 Trial design of Isolator…………………………………………….30

3.4 Base isolator selection……………………………………………...32

3.5 Preliminary design of base isolator……………………………….32

3.6 Analysis of base isolated structure………………………………..33

3.6.1Response spectrum analysis…………………………………….....33

3.6.2 Time history analysis…………………………………………...…35

3.7 SAP 2000…………………………………………………………....35

3.8 UBC-97 Base isolation design specification……………………....38

4. NUMERICAL STUDY …………………………………………….39

4.1 General study parameters………………………………………...39

4.2 Numerical Example……………………………………………….42

4.2.1 Dimensions of 3D frame…………………………….………...….42

4.2.2 Isolator properties………………………………………………...46

4.2.3 Seismic loading……….…………………………………………..48

4.3 Numerical study : Steps followed…………………………………49

5. RESULTS & DISCUSSION………………………………………..50

5.1 Analysis results………………….………………………………....52

5.2 Discussion of results…………………….…………………………65

6. CONCLUSION……………………………………………………..68

7. SCOPE FOR FUTURE RESEARCH…………………………….69

REFERENCES……………………………………………………….70

- LIST OF FIGURES

Chapter 1 : Introduction and Literature Review

Fig. 1.1 (a,b,c,d) : Conventional Retrofitting techniques……………...3-4

Fig. 1.1a) Additional foundations……………………………………….3

Fig.1.1b) Additional shear walls……………………………………...…3

Fig. 1.1c) Jacketing……………………………………………………...4

Fig.1.1d) Additional column………………………………………….....4

Fig. 1.2a) Deformation before base isolation……………………………7

Fig.1.2b) Deformation after base isolation………………………………7

Fig. 1.3 ) Period shifting in case of base isolation……………………….9

Fig.1.4). A typical elastomeric bearing………………………………...11

Fig. 1.5 ) Schematic diagram of (a)LRB  (b) NZ System……………...11

Fig.1.6) Low friction bearing device…………………………………...13

Fig. 1.7 schematic diagram of (a) P-F & (b) R-FBI system……………13

Fig. 1.8 diagram depicting mechanism of FPS…………………………14

Fig. 1.9 schematic diagram of (a) EDF & (b) SR-F system……………15

Fig. 1.10 Installation of temporary steel props………………………....20

Fig 1.11: Saw cut through the concrete column………………………..22

Fig 1.12: Removal of concrete block……………………………………22

Fig 1.13:Steel jackets replacing the discontinued reinforcing bars……..23

Fig 1.14: Fireproofing insulation………………………………………..24

CHAPTER 3. BASE ISOLATION

Fig. 3.1) Idealized Force Deflection behavior  of an Isolator…………...27

Fig. 3.2)Force Deflection curve for a Linear Base Isolator……………..27

Fig. 3.3.)Force Deflection curve for non linear Isolator………………...29

Fig. 3.4) Spectral Plot for different damping ratios(according to IS 1893-2002)………………………………………………………………….....31

Fig.3.5) Nonlinear Behavior of  NLlink element in SAP 2000………....37

CHAPTER 4. NUMERICAL STUDY

Fig. 4.1: Plan of  the L-shaped frame showing Xi & Yi………………..41

Fig. 4.2: 3D view of L-shaped frame…………………………………...43

Fig. 4.3:Plan  view of the L-shaped  frame……………………………..44

Fig. 4.4: Elevation of the fixed L-shaped frame.

(at section BB in plan)…………………………………………………..45

Fig. 4.5: Elevation of isolated frame (at section AA in plan)…………..47

Fig. 4.6: Plot of El-Centro time history NS component -1940…………48

CHAPTER 5. results & Discussion

Fig.5.1 : plan specifying location of columns C1,C2,

C3,C4,C5 & C6………………………………………………………...51

Fig.5.2 (a)  1st mode shape for fixed base  frame : 3D view…………...56

Fig.5.2 (b)  1st mode shape for fixed base  frame : top view…………...57

Fig.5.2 (c)  1st mode shape for uniform isolated frame : top view……..57

Fig.5.2 (d)  1st mode shape for uniform isolated frame : 3D view……..58

F ig.5.2 (e) 1st mode shape for different (random) isolator

stiffness frame : 3D view…………………………………………….....59

Fig.5.2 (f) 1st mode shape for isolator stiffness in mass

ratio frame : 3D view…………………………………………………...60

Fig.5.3 (a) top displacement time history for frame

resting on fixed base…………………………………………………….61

Fig.5.3 (b) top displacement time history for frame

resting on isolators of uniform stiffness………………………………...61

Fig.5.3 (c) top displacement time history for frame resting on isolators of randomly different stiffness……………………………………………..62

Fig.5.3 (d) top displacement time history for frame resting on isolators of different stiffness in proportion to load coming on the column…………………………………………………………………..62

Fig.5.4 (a) BMD at sec. BB for fixed frame subjected to  response spectrum (IS  1893:2002) loading………………………………………63

Fig.5.4 (b) BMD at sec. BB for fixed frame subjected to time history (El-Centro) loading…………………………………………………………64

LIST OF TABLES

CHAPTER 5. results & Discussions

- Table 5.1: Fundamental time period in sec.:(modal analysis:

- for 1st mode)……………………………………………………………52

- Table 5.2 : Total Base shear (in KN)…..……………………………….52

- Table 5.3 : Base shear (in KN) in ground storey columns……………...53

- Table 5.4 : Frequency ratio & eccentricity values (in mm.)…………....54

- Table 5.5 : Base and top displacements (maximum) : (in mm.)………..55

- Table 5.6 : Maximum Inter storey Drift values (in mm)……………….55

- Table 5.7 : Bending moment (maximum) values :  (in KN-m)………...55

LIST OF SYMBOLS & ABBREVIATIONS

BMD =  Bending Moment Diagram

C = Damping matrix

C0 = Damping coefficient for bearing

Ce = Linear effective damping

CG = Centre of gravity

CR = Centre of rigidity

DBE = Design Basis Earthquake

e = Eccentricity

EDF = Electricide De France

F = Force

F+ = Maximum force in positive direction

F-  = Maximum force in negative direction

Fmax =  Absolute maximum force

FPS = Friction Pendulum System

g = Gravitational acceleration

I = Moment of inertia

K = Stiffness matrix

K0 = Stiffness of  bearing

Kb = Stiffness of base isolator

Kbx = Base isolator stiffness in X – direction

Kby = Base isolator stiffness in Y- direction

Ke = Linear effective stiffness

Kx = Effective stiffness in X-direction

Ky = Effective stiffness in Y-direction

Kɵ = Effective torsional stiffness

LRB = Laminated Rubber Bearing

M = Diagonal mass matrix

MCE = Maximum Capable Earthquake

NL Link = Non Linear link

NS Component = North South Component

NZS = New Zealand System

P-F System = Pure Friction System

R = Response reduction factor

r = Radius of gyration

R-FBI = Resilient Friction Base Isolator

RCC = Reinforced Cement Concrete

Sa = Spectral Acceleration

SR-FBI = Sliding Resistance  Friction Base Isolator

T = Fundamental Time period

Tn = Fundamental Time period of structure

Tnb = Fundamental Time period of  base isolator

UBC = Uniform building Code

X = Displacement matrix

X’ = Velocity matrix

X’’ = Acceleration matrix

Xi = Distance of  ith isolator from CR along X-axis

Yi = Distance of  ith isolator from CR along Y-axis

Z = Zone factor

z = Internal hysteretic variable

Δ = Deflection

Δ+ = Maximum deflection in positive direction

Δ- = Maximum deflection in negative direction

Δmax = Absolute maximum deflection

µ = Coeffecient of friction

ξ b = Damping of base isolator

ω = Frequency ratio

ωn = Fundamental frequency of  structure

ωnb = Fundamental frequency of  base isolator

ωx  = Lateral frequency in X-direction

ωy = Lateral frequency in Y direction

ωɵ = Torsional frequency

3D = Three Dimensional

- ABSTRACT

The need for structural and seismic retrofitting arises due to many reasons like loss of strength of building materials, damage in structures due to earthquakes, inadequate design of structures etc. After the revision of seismic zones in India, existing structures are supposed to be seismically retrofitted for higher expected earthquake forces.

Structural retrofitting for earthquakes can be done either by addition of lateral force restraining systems (seismic) or by addition of lateral force reducing systems (aseismic) . Again aseismic retrofitting can be of two types, namely passive and active devices. Base isolation is a passive method for aseismic retrofitting

The concept of base isolation suggests an effective measure to reduce the earthquake induced lateral forces on the building. The idea behind the concept of base isolation is quite simple :

In this approach, the building or structure is decoupled from the horizontal components of the earthquake ground motion by interposing a layer with low horizontal stiffness between the structure and the foundation. This layer gives the structure a fundamental frequency that is much lower than its fixed-base frequency and also much lower than the predominant frequencies of the ground motion.

Technique of base isolation, if applied, can substantially reduce the base shear by increasing the time period of vibration in isolation mode.

There are two basic types of isolation systems:

- The system that has been adopted most widely in recent years is typified by the use of elastomeric bearings.

- The second basic type of isolation system is typified by the sliding system.

In this dissertation work an L- Shape , 7 story , 3 bay , 3D  frame is being modeled  in SAP 2000 VER. 11 (Advanced) & analyzed for four different cases viz :  fixed, uniformly isolated (i.e. isolators of same stiffness are being used), isolators with different stiffness &  then  for isolator stiffness in the ratio of load coming on individual column.

These frames are being analyzed using response spectrum (IS 1893-2002) and time history (EL-Centro, NS Component) in SAP 2000 and then the results are being compared, and it is being observed that one can not draw much advantage  by using isolators of different stiffness ( i.e. stiffness of isolators provided are in ratio of the load coming on the columns ). And after analysis various displacement time histories and base shear time histories obtained are presented here & results for different cases are being compared.

The study suggests that one should better go for isolators of uniform stiffness rather than going for non-uniform stiffness. Because little advantage is drawn but the practical feasibility of getting manufactured  the isolators of different stiffness is very low and it will not be economical too. Although considering relative top displacement, non-uniformity in mass ratio is a better option, as observed in the study. But the major parameter governing the torsional behavior of structure is the “Frequency ratio values” & the study suggests that torsional coupling will be lesser in the case when isolators of uniform stiffness are being used as in this case the value of frequency ratio obtained is more than that for other cases. So keeping in mind  the meager  gain (in terms of storey drifts) & low feasibility, it is advisable to go for uniform stiffness isolators, specially when torsional coupling is of  much concern. One can go for different stiffness for isolators to be used for exterior & interior columns, which is a quite common practice. But going for ‘n’ different values of isolator stiffness will not be feasible & economical.