# A

## PROJECT REPORT

# ON

Embedded Control of Bottle Filling Prototype Plant

A Major Project Submitted to

Delhi College of Engineering, Delhi

In partial fulfillment of the requirements for the award of degree of

MASTER OF ENGINEERING

IN

ELECTRICAL ENGINEERING (CONTROL & INSTRUMENTATION)

Submitted by

Neelu Nagpal (18/C&I/05)

University Roll No .10701

## Under the guidance of

## Prof. Pramod Kumar

HOD, Deptt. Of Electrical Engg., DCE, Delhi

Department Of Electrical Engineering

DELHI COLLEGE OF ENGINEERING

University Of Delhi,2008

DELHI COLLEGE OF ENGINEERING

DELHI-110042

##### Department of Electrical Engineering

###### CERTIFICATE

This is to certify that Neelu Nagpal student of final Year M.E. Electrical Engineering of Delhi College of Engineering during the session 2007—2008 has successfully completed the project work on ’Embedded control of Bottle Filling Prototype Plant’ and has submitted a satisfactory report on this volume as per requirement of university which may accepted for the partial fulfillment of the degree of Master of Engineering in Electrical Engineering(Controls and Instrumentation).

Prof. Pramod Kumar

Project Guide

Head of Department,

Department of Electrical Engineering

D.C.E, Delhi.

Acknowledgement

Prof. Pramod Kumar initiated the founding ideas behind this thesis and provided insight and valuable information into many of the problems that were encountered. His positive attitude throughout the duration of this thesis always put things into perspective whenever difficulties arose. This thesis would not have been possible without him.

Many other people have contributed directly or indirectly to this thesis. Some acting as sounding boards, sources of information, or providing resources that contributed to the development of my electronics. While their individual contributions have been small, their efforts have still been greatly appreciated.

Finally, I would like to thank my family and friends for keeping me sane.

Neelu Nagpal

ABSTRACT

This project consists of three parts:

- Electronics- includes 8051 Microcontroller, driver circuits for driving moving mechanism and for operating sensors.

- Programming- The 8051 Development Kit is a multipurpose kit. It can be used to run various 8051 programs and burn them into the microcontroller. It can also be used in practical Robotic/Automation applications where implementation of Actuators and Sensors may be required. IDE software is used for the interfacing purpose

- Mechanical- includes moving mechanism (conveyor belt with a motor system) and a Robotic arm to pick and place the bottle. It also includes pumping mechanism that does the operation of filling the liquid into the bottle.

For the proper functioning of the plant, Light Dependent Resistors (LDRs) using LASER as the input beam has to be installed at various places. I have decided to put on three of them; each at the starting point, filling point and at the end.

The foremost requirement of our project is the 8051 microcontroller, which fulfills the

Criteria of making the project autonomous. Despite it is relatively old age, the 8051 is one of the most popular microcontrollers in use today. Many derivative microcontrollers have since been developed that are based on and compatible with the 8051. Thus, the ability to program an 8051 is an important skill for anyone who plans to develop products that will take advantage of microcontrollers. Another important part of this project is the robotic arm that is used at the end point of the conveyor belt to pick and place the bottle the filled bottle from belt. The proper operation of robotic arm cannot be done without the use of the H-Bridge.

List of Abbreviations

- DOF  – Degree of Freedom

- IDE – Integrated Development Environment.

- LASER – Light amplification by stimulated emission of radiation

- LDR – Light detecting Resistor

- LED – Light emitting diode

- UART- Universal Asynchronous Receiver Transmitter or belt to pick up

Table of Contents							      Page

- Introduction   								          1

1.1 Embedded Control& its need  					          2-3

1.2 Advantages & Important features of Embedded Control	   	          2-3

1.3 Components of  Embedded control	        	   		          4

1.4 Microcontroller used for Bottle filling plant		           	           5

1.4.1 General Overview	           					           5-6

1.4.2 Block Diagram 						           7

1.4.3 Description of 89S52 MCU				           8

1.4.4 Atmel  AT89S52 Pin out and Description			         10

1.4.5 Software overview	             				         11-12

1.5  Objective of  the Project	           					          13

1.6 Dissection of the Project Report	           				          14

2.  Problem Definition	           						          15

3.  Project Layout	           							          16

4.  Literature Review						           	          17

4.1 About Microcontroller					           	          18

4.2 Operation of Microcontroller					           	          18

4.3 Inside Microcontroller					           	         19-24

4.3.1 Program memory

4.3.2 Data memory

4.3.3 EEPROM memory

4.3.4 Special Function Registers

4.3.5 Program Counter

4.3.6 CPU

4.3.7 Input Output Ports

4.3.8 Timer

4.3.9 Register

4.3.10 Watchdog Timer

4.3.11 Power Supply

4.3.12 Serial Communication

4.4 Instruction Set	           						     25-26

4.5 Stepper Motor	           						     27-28

4.6 Robot		           					                 29-30

5. Design Consideration	           						     31-33

6. Hardware Implementation          						         34

6.1 Project Views	           						         35

6.2 Implementation	           						         36

6.2.1 Moving System	           						         36

6.2.1.1 Conveyor Belt- Stepper Motor and Driver circuit	         37

6.2.1.2 Robotic Arm- Dc motor and H Bridge 			         40

6.2.2 Sensing System	           					                     51-52

6.2.2.1 LDR	           						         53

6.2.2.2 LASER	           						         54

6.2.2.3 OpAmp Circuit	           					         55

6.2.2.4 Reed Switch	           					         56-57

6.2.3 Controlling System- Basic 8051 and Power supply		         58

7. Software Implementation	           						         60

7.1 Using 8051 IDE Version	           				                     61-62

7.2 Sample window of project	           			                     63-64

7.3 Burn MCU using Up loader	           				         65-67

8. Working	           								         68

8.3 flow Chart	           							         69

8.1 Circuit Diagram	           						         70

8.2 Working	           							         71

9.  Expected Results           						         72

10. Conclusions 		           						         73

11. Future Scope of work	           						         74

References

Appendix