- Acknowledgement

I would like to acknowledge those people and institute who have given me the knowledge and light of guidance which has helped me to wade through the darkness. Here, I  express my heartily and sincere gratitude and indebtedness to Mr. A. ARORA, lecturer of Mechanical Engineering Department, Delhi College of Engineering, Delhi for his valuable guidance and wholehearted cooperation. He has a special place in my heart for many reasons but to be limited, he is the one who inspires me to move ahead in life with persistence and endurance.

My heartily thanks to all of my Professors for their expertise and all rounded personality they have imparted me.

My special thanks go to my parents who have given me the strength, love and care to carry out this course successfully.

Gaurav Kumar Shukla

Abstract

The classical substances like CFCs and hydrochlorofluorocarbons (HCFCs) used as working fluids in the vapour compression plants have to be replaced by new substance because of high ozone depletion potential and their global warming potential. In this thesis the analysis for transcritical vapour compression refrigeration system and to increase the performance of the system addition of absorption refrigeration system cycle to transcritical vapour compression refrigeration cycle has been suggested. A computational model for the parametric investigation of these systems has been developed. The analysis involves the determination of effects of pressure of gas cooler, internal heat exchanger, efficiency of compressor, absorber, condenser and evaporator temperatures on the energetic and exergetic performance of these systems. The performance parameters computed are coefficient of performance, exergy destruction, and exergetic efficiency. The results indicate that the pressure of gas cooler yielding to maximum COP is termed optimum, which are around 122 bar. The maximum COP after optimisation of pressure is increased to 14% and exergy destruction is reduced to 8.5% and after addition of the absorption refrigeration system to optimum transcritical system, the COP is again increased to 46% and exergy destruction is reduced by 13.7%. Exergetic efficiency of the combined system is also increased around 46%. The effect of efficiency of compression on COP is much more as compared to that of effectiveness of internal heat exchanger on COP. Compressor, gas cooler and expansion valve of transcritical system play a major role on the exergy destruction of the combined system they are responsible for around 67% of exergy destruction.

Key word – Transcritical vapour compression refrigeration system, Vapour absorption refrigeration system

Content

| Acknowledgement |  |  | i |
| --- | --- | --- | --- |
| Abstract |  |  | ii |
| Contents |  |  | iii |
| List of figure |  |  | vi |
| List of table |  |  | ix |
| Nomenclature |  |  | x |

| Chapter 1 |  |  | Introduction |  |
| --- | --- | --- | --- | --- |
|  | 1.1 |  | Overview of ozone layer depletion and global warming | 3 |
|  |  | 1.1.1 | Ozone layer depletion | 3 |
|  |  | 1.1.2 | Global Warming | 5 |
| Chapter 2 |  |  | Literature Review |  |
|  | 2.1 |  | Introduction | 7 |
|  | 2.2 |  | Literature Review | 7 |
|  | 2.3 |  | Conclusion of literature review | 12 |
|  | 2.4 |  | Objective of thesis | 13 |
| Chapter 3 |  |  | Thermodynamic analysis |  |
|  | 3.1 |  | System description | 14 |
|  |  | 3.1.1 | Transcritical Vapour compression refrigeration system | 14 |
|  |  | 3.1.2 | Single effect water lithium bromide Vapour absorption system | 16 |
|  |  | 3.1.3 | Combined transcritical and absorption system | 18 |
|  | 3.2 |  | Mass conservation of combined system | 19 |
|  | 3.3 |  | Energy analysis of combined cycle | 20 |
|  | 3.4 |  | Exergy analysis | 22 |
|  | 3.5 |  | Simulation study | 25 |
|  | 3.6 |  | Validation of result | 26 |
|  | 3.7 |  | Solution mythology in engineering equation solver (EES) | 27 |
| Chapter 4 |  |  | Result and discussion |  |
|  | 4.1 |  | Optimisation of gas Cooler pressure of transcritical vapour compression refrigeration system | 28 |
|  | 4.2 |  | Input parameter | 31 |
|  | 4.3 |  | Computer program and its output | 32 |
|  | 4.4 |  | Operating conditions for the combined cycle | 33 |
|  | 4.5 |  | Comparison of results | 34 |
|  | 4.6 |  | Energy and exergy analysis of combined system | 35 |
|  | 4.7 |  | Effect of variation in gas cooler pressure | 36 |
|  | 4.8 |  | Effect of variation in temperature of refrigerant leaving the gas cooler | 38 |
|  | 4.9 |  | Effect of variation in effectiveness of the internal heat exchanger | 39 |
|  | 4.10 |  | Exergy destruction in different component of a transcritical Vapour compression refrigeration system | 40 |
|  | 4.11 |  | Effect of variation of temperature of cooling media at the outlet of gas cooler | 40 |
|  | 4.12 |  | Effect of variation in effectiveness of solution heat exchanger | 41 |
|  | 4.13 |  | Effect of variation in absorber temperature | 41 |
|  | 4.14 |  | Effect of variation in condenser temperature | 42 |
|  | 4.15 |  | Effect of difference in absorber and condenser temperatures | 43 |
|  | 4.16 |  | Effect of variation in temperature difference of cooling media at evaporator (T22-T23). | 43 |
|  | 4.17 |  | Effect of variation in temperature difference of cooling media at condenser (T27-T26) | 44 |
|  | 4.18 |  | Effect of variation in temperature difference of cooling media at absorber (T25-T24) | 44 |
|  | 4.19 |  | Exergy destruction of each component of combined system | 45 |
|  | 4.20 |  | Total exergy destruction of different system | 45 |
| Chapter 5 |  |  | Conclusions | 61 |
| Chapter 6 |  |  | Scope for future work | 63 |
|  |  |  | References | 64 |
|  |  |  | Appendix | 67 |

List of figure

| S No. | Figure No. | Title | P. No. |
| --- | --- | --- | --- |
| 1 | 3.1 | Schematic diagram of Transcritical vapour compression refrigeration system | 15 |
| 2 | 3.2 | P- h diagram of transcritical system | 15 |
| 3 | 3.3 | Schematic diagram of absorption refrigeration system | 17 |
| 4 | 3.4 | Representation of absorption cycle on ln P – 1/T diagram | 17 |
| 5 | 3.5 | Schematic diagram of a combined Transcritical and Vapour Absorption System | 18 |
| 6 | 3.6 | Mass conservation of combined system | 19 |
| 7 | 4.1 | Variation of COP versus pressure of gas cooler | 46 |
| 8 | 4.2 | Variation of COP versus pressure of gas cooler | 46 |
| 9 | 4.3 | Variation of COP versus pressure of gas cooler | 47 |
| 10 | 4.4 | Variation of Exergetic efficiency versus Pressure of gas cooler | 47 |
| 11 | 4.5 | Variation of COP and exergetic efficiency of system versus pressure of gas cooler | 47 |
| 12 | 4.6 | Variation of COP versus pressure of gas cooler | 48 |
| 13 | 4.7 | Variation of exergetic efficiency versus pressure of gas cooler | 49 |
| 14 | 4.8 | Variation of total exergy destruction and effectiveness of gas cooler versus pressure of gas cooler | 49 |
| 15 | 4.9 | Variation of waste energy available at gas cooler, compressor work, and waste energy available per kW of compressor work versus pressure gas cooler | 50 |
| 16 | 4.10 | Variation of total exergy destruction and mass of refrigerant versus pressure of gas cooler | 50 |
| 17 | 4.11 | Variation on COP and exergetic efficiency versus temperature of refrigerant after gas cooler | 51 |
| 18 | 4.12 | Variation on effectiveness of gas cooler and total exergy destruction of transcritical system versus temperature of refrigerant after gas cooler | 51 |
| 19 | 4.13 | Variation on energy available at gas cooler and mass of refrigerant versus temperature of refrigerant after gas cooler | 52 |
| 20 | 4.14 | Variation of COP and exergetic efficiency of transcritical system versus effectiveness of internal heat exchanger | 50 |
| 21 | 4.15 | Variation of total exergy destruction of system and waste energy available at gas cooler and versus effectiveness of internal heat exchanger. | 52 |
| 22 | 4.16 | Exergy destruction of different component of transcritical vapour compression refrigeration system | 53 |
| 23 | 4.17 | Percentage of exergy destruction of different component of transcritical vapour compression refrigeration system | 54 |
| 24 | 4.18 | Variation of combined COP and refrigeration capacity of LiBr cycle versus temperature of cooling media at outlet of gas cooler | 54 |
| 25 | 4.19 | Variation of mass of refrigerant in absorption cycle and effectiveness of gas cooler versus temperature of cooling media outlet of gas cooler | 55 |
| 26 | 4.20 | Variation of COP and exergetic efficiency of combined system versus effectiveness of solution heat exchanger | 55 |
| 27 | 4.21 | Variation of COP and exergetic efficiency versus absorber temperature | 56 |
| 28 | 4.22 | Variation of total exergy destruction and effectiveness of generator versus absorber temperature | 56 |
| 29 | 4.23 | Variation of COP and exergetic efficiency versus condenser temperature | 57 |
| 30 | 4.24 | Variation of COP and exergetic efficiency of combined system versus difference in absorber and condenser temperature | 57 |
| 31 | 4.25 | Variation of exergy destruction and mass of cooling media versus difference in inlet and outlet temperature of cooling media at evaporator | 58 |
| 32 | 4.26 | Variation of exergy destruction and mass of cooling media with respect to the difference in inlet and outlet temperature of cooling media at condenser | 58 |
| 33 | 4.27 | Variation of exergy destruction and mass of cooling media versus difference in inlet and outlet temperature of cooling media at absorber | 59 |
| 34 | 4.28 | Exergy destruction of each component of combined system | 60 |
| 35 | 4.29 | Total exergy destruction in different system | 60 |

List of table

| S. No. | Table No. | Title | P. No. |
| --- | --- | --- | --- |
| 1 | 1.1 | Source - Market transformation programme supporting UK government policy on sustainable products | 6 |
| 2 | 3.1 | Comparison of present results with results of Fartaj et al. (2004) for validation of result. | 26 |
| 3 | 4.1 | Operating conditions for the combined cycle | 33 |
| 4 | 4.2 | Comparison of results for COP and exergy destruction of present system with numerical data given by Fartaj et al. (2004) | 34 |
| 5 | 4.3 | Energy and exergy analysis of combined system | 35 |

Nomenclature

| Abs | Absorber |
| --- | --- |
| C | Condenser |
| COP | Coefficient of performance |
| Comp | Compressor |
| Comb | Combined system |
| CO2 | Carbon dioxide |
| Cp | Specific heat (kJ/kg K) |
| ED | Exergy destruction (kW) |
| E | Evaporator |
| EV | Expansion valve |
| G | Generator |
| GC | Gas cooler |
| GDP | Global warming potential |
| h | Enthalpy (kJ/kg) |
| IHE | Internal heat exchanger |
| LiBr | Lithium bromide |
| m | Mass flow rate of refrigerant in transcritical cycle (kg/s) |
| ODP | Ozone layer depletion potential |
| P | Pressure (KPa) |
| Q | Heat flux (kW) |
| RC | Capacity of refrigeration system (TR) |
| R-744 | Carbon dioxide |
| S | Entropy (kJ/kg K) |
| SHE | Solution heat exchanger |
| T | Temperature (0C) |
| TED | Total exergy destruction |
| VAR | Vapour absorption refrigeration |
| W | Mechanical work rate (kW) |
| WEAG | Waste energy available at gas cooler (KW) |
| X | Mass faction of lithium bromide (%) |
| Subscripts |  |
| 1, 2, 3. . | States as shown in Figs |
| a | Vapour absorption refrigeration system |
| abs | Absorber |
| c | Condenser |
| cm | Cooling medium |
| comp | Compressor |
| comb | Combined system |
| et | Evaporator of transcritical cycle |
| ea | Evaporator of absorption cycle |
| ev | Expansion valve |
| ex | Exergetic |
| f | Circulation rate( kg/kg of vapour) |
| g | Generator |
| gc | Gas cooler |
| ihe | Internal heat exchanger |
| o | Ambient state |
| r | Refrigerant in transcritical cycle |
| ra | Refrigerant in absorption cycle |
| sev | Solution expansion valve |
| she | Solution heat exchanger |
| s | Strong solution |
| sol | Solution of Libr and water |
| t | Transcritical system |
| w | Weak solution |
| Greek symbols |  |
| φ | Exergy (kJ/kg) |
| η | Efficiency |
| ∆φ | Exergy destruction |
|  | Effectiveness |