PIC MICROCONTROLLER BASED

DATA ACQUISITION SYSTEM

A dissertation submitted in partial fulfilment of the Requirement

for the degree of

MASTER OF ENGINEERING

IN

ELECTRICAL ENGINEERING

(CONTROL AND INSTRUMENTATION)

BY

VIJAY KUMAR SHARMA

(ROLL No. 10703)

Under the guidance of

Prof. PARMOD KUMAR

Department of Electrical Engineering

Delhi College of Engineering

(University of Delhi)

New Campus, Bawana Road, Delhi-42

Year 2008

CERTIFICATE

This is to certify that the dissertation entitled “PIC Microcontroller  based data acquisition SYSTEM” submitted by  Mr. Vijay Kumar Sharma, University Roll No. 10703 towards partial fulfilment of the requirement for the award of  Master of Engineering degree in Control and Instrumentation,  Electrical Engg. Department, Delhi college of Engineering, has been carried out under my supervision & guidance.

I wish him all the best for his bright future and successful career.

Date:

Prof. Parmod Kumar

Department of Electrical Engg.,

Delhi College of Engineering,

University of Delhi,

Delhi - 110042

ACKNOWLEDGEMENT

I would like to extend my sincere gratitude and thanks to my guide Prof. Parmod Kumar, Electrical Engg. Deptt., Delhi College of Engineering, for his assistance and invaluable guidance towards the progress of this dissertation.

I am also thankful to Mr. Karan Sigh, Control and Instrumentation Lab., Delhi College of Engineering.

Date:                                                          Vijay Kumar Sharma

M. E. (C&I)

University Roll No.   10703

ABSTRACT

Microcontrollers are a new concept of integrated circuits based on a multifunctional architecture. The majority of microprocessors require a lot of discrete components and ICs to execute a relative simple function, resulting in large and complex hardware. This is based on a fact that the microprocessors must be flexible, in the application point of view. This architecture is difficult to update and requires a lot of electronics knowledge and low level programming.

Incorporating some basic functions inside of a microcontroller, the hardware can be quite simple and with low cost as well. Some of these basic functions are: A/D converter, comparator, PWM, serial communication, internal clock, RAM/EPROM memory, flash memory and programmable I/O. Some microcontrollers incorporates all of these functions in a single chip, the additional discrete components are minimal.

Another advantage of the recent microcontroller is the programming process and the language used. The software can be made with high level languages, C++, Basic or low level language (assembly). To upload the software into the microcontroller, just a PC and an external board is required.

Actually a large number of different microcontrollers are available for different applications and from different manufacturers: Zilog, National, Motorola and Microchip. The Microchip developed the PIC Microcontroller family, very popular now a days.

We have used PIC 16F877 microcontroller here. A data acquisition system using this PIC 16F877 microcontroller is shown in this dissertation. Three (out of the eight available) analog channels are used here. A programme in C is written and compiled via PCW C-compiler. The programme is such that, different values for the three measured quantities are preset in the programme itself. The instant values of the temperature, light and input AC voltage are displayed on the LCD and compared with preset values. The Microcontroller outputs the comparison result on programmed port pins. This output signal may be used for effecting any further activity e.g. when the light in the surrounding area diminishes a lamp may be switched on or a fan may be switched on if temperature exceeds the preset values.

Important PIC 16F877A Microcontroller features are outlined in this dissertation. PCW C-compiler features and datasheets of different hardwares are also included.

CONTENTS

CHAPTER 1: INTRODUCTION

1.1                 Introduction of the project                                                                 1

1.2                 Dissection of dissertation                                                                   2

CHAPTER 2: PIC 16F877 MICROCONTROLLER

2.1                Introduction                                                                                         3

2.2                What is PIC Microcontroller                                                               4

2.3                Advantages of PIC Microcontroller                                                    4

2.4                Selecting PIC Microcontroller                                                            5

2.5                Characteristics of PIC Microcontroller                                               7

2.6                Architecture of PIC Microcontroller                                                   9

2.7                Pin diagram                                                                                       10

2.8                Instructions                                                                                        16

CHAPTER 3: ANALOG-TO-DIGITAL CONVERTER (A/D) MODULE

3.1                Introduction	                                                                         18

3.2                ADC Registers 	                                                                                  18

3.3                Requirements for AD Conversion                                                     21

CHAPTER 4: TIMER 1 MODULE

4.1                 Introduction                                                                                      23

4.2                 TIMER 1 Control Register                                                               23

CHAPTER 5:  LCD INTERFACING WITH PIC MICROCONTROLLER

5.1                 Introduction                                                                                       25

5.2                 Instructions for LCD write                                                               27

5.3                 LCD Interfacing in 4-bit Mode                                                        30

CHAPTER 6: PCW C-COMPILER

6.1                 Introduction                                                                                      33

6.2                 PCW Overview                                                                                36

6.3                 Some useful built-in-functions / pre-processor directives                44

CHAPTER 7: EXPERIMENTAL WORK

7.1                Flow charts                                                                                        51

7.2                Circuit diagram                                                                                  56

7.3                Programme                                                                                        60

7.4                Programmer                                                                                      70

CHAPTER 8: CONCLUSION AND FURTHER SCOPE

8.1               Conclusion                                                                                          71

8.2               Future scope                                                                                       71

DATASHEETS                                                                                           72 to 87

References                                                                                                       88