## Appendix-A

MATLAB CODE DEVELOPED FOR ALGORITHM NO. 1

b=input('Enter the order of the numerator equation:')

a=input('Enter the coefficient of numerator equation:')

c=input('Enter the order of denominator equation:')

d=input('Enter the coefficient of denominator equation:')

num1=a

den1=d

%************CACULATE THE ROOTS OF HIGHER ORDER SYSTEM***************

den=roots(den1)

k1=max(den)

k2=min(den)

%***********CALCULATE POLE CENTROID OF HIGHER ORDER SYSTEM***********

disp('**************POLE CENTROID OF HIGHER ORDER SYSTEM***********')

pc=mean(den)

%***********CALCULATE STIFFNESS OF HIGHER ORDER SYSTEM***********

disp('**************STIFFNESS OF HIGHER ORDER SYSTEM***********')

s=k2/k1

c1=input('Enter the order of lower order equation:')

p=c1

%*****************CALCULATE THE VALUE OF Q:**************

Q=0;

for i=1:(p-2)

Q=Q+i

end

[Q]

%************DETERMINATION OF EIGEN SPECTRAL POINT OF LOS**************

A=[s*(p-1)+1 Q;(1-s) (1-p)]

B=[pc*p ;0]

C=[inv(A)*B]

np=C(1)-(C(2)*(p-1))

D=conv([1  -np],[1  -np-C(2)])

for i=1:(p-2)

E=[1 -np-((i+1)*C(2))]

D=conv(D,E)

End

%************DENOMINATOR OF LOS***************

den2=D

%**************CALCULATE THE VALUE OF e(0....i)*************

[q,r]=deconv(den1,den2)

%**************CALCULATE NUMBER OF ROWS& COLUMNS***********

[m,no]=size(num1)

%***************CHANGE ARRAY IN REVERSE ORDER***************

j=no;

for i=1:1:no

if i>=j

break

else

temp=num1(i);

num2(i)=num1(j);

num2(j)=temp;

j=j-1;

end

end

[num2]

if (p+1>no)

for i=(no+1):1:p+2

num2(i)=0;

end

end

[num2]

#### %*************CALCUTATE COLUMN OF  e(0.......i)***********

#### [m,n1]=size(q)

#### %*************CHANGE ARRAY IN REVERSE ORDER***************

#### j=n1;

#### for i=1:1:n1

#### if i>=j

#### break

#### else

#### temp=q(i);

#### q(i)=q(j);

#### q(j)=temp;

#### j=j-1;

#### end

#### end

#### [q]

if (p>n1)

for i=(n1+1):1:p+1

q(i)=0;

end

end

[q]

% ***************APPLY ROUTH RECURRENCE ALGORITHM FORMULA***********

v=0;

for  i=1:1:p;

x(i)=num2(1)/q(1)

for  j=1:1:p-v

num2(j)=num2(j+1)-(x(i)*q(j+1));

end

v=v+1;

end

%**************CALCULATE COLUMN OF NUMERATOR OF LOS******************

[m,n2]=size(x)

%**************CHANGE ARRAY IN REVERSE ORDER***************

j=n2;

for i=1:1:n2

if i>=j

break

else

temp=x(i);

x(i)=x(j);

x(j)=temp;

j=j-1;

end

end

[x]

disp('**************GIVEN HIGHER ORDER TRANSFER FUNCTION IS*******************')

sys1=tf(num1,den1)

disp('**************REDUCED ORDER TRANSFER FUNCTION IS*******************')

sys2=tf(x,den2)

t=0:0.001:8;

[y,x0,t]=step(x,den2,t);

[z,x0,t]=step(num1,den1,t);

#### % *********************PLOT THE STEP RESPONSE CURVE OF HIGHER ORDER AND LOWER ORDER TRANSFER FUNCTION******************

#### plot(t,y,'r',t,z,'k');

#### grid

#### title('step response');

#### xlabel('TIME(sec)');

#### ylabel('AMPLITUDE');

#### %******************CALCULATE THE VALUE OF RISE TIME, PEAK TIME, MAX. OVERSHOOT AND SETTLING TIME FOR HOS AND LOS*************************

#### r1=1;

#### while y(r1)<0.9;

#### r1=r1+1;

#### end

#### disp('**************RISE TIME OF THE HIGHER ORDER SYSTEM IS****************')

#### rise_time1=(r1-1)*0.001

#### [ymax,tp1]=max(y);

#### disp('********************PEAK TIME OF THE  HIGHER ORDER SYSTEM************************')

#### peak_time1=(tp1-1)*0.001

#### disp('******************* MAX. OVERSHOOT OF THE HIGHER ORDER SYSTEM*****************')

#### max_overshoot1=ymax-1

#### s=1001;

#### while y(s)>0.98&y(s)<1.02;

#### s=s-1;

#### end

#### disp('****************SETTLING TIME OF THE  HIGHER ORDER SYSTEM*****************')

#### settling_time1=(s-1)*0.001

#### disp('*****************RISE TIME OF THE REDUCED ORDER SYSTEM********************')

#### r2=1;

#### while z(r2)<0.9;

#### r2=r2+1;

#### end

#### rise_time2=(r2-1)*0.001

#### [zmax,tp2]=max(z);

#### disp('*********************PEAK TIME OF THE REDUCED ORDER SYSTEM*********************')

#### peak_time2=(tp2-1)*0.001

#### disp('*********************MAX. OVERSHOOT OF THE REDUCED ORDER SYSTEM********************')

#### max_overshoot2=zmax-1

#### s=1001;

#### while z(s)>0.98&z(s)<1.02;

#### s=s-1;

#### end

#### disp('**************SETTLING TIME OF THE REDUCED ORDER SYSTEM******************')

#### settling_time2=(s-1)*0.001

%******************CALCULATE THE VALUE OF INTEGRAL SQUARE ERROR*************************

syms s t x w

denn=conv([1 0],den1)

den1n=poly2sym(denn)

num1n=poly2sym(num1)

sr1=ilaplace(num1n/den1n)

syms s t x w

dena=conv([1 0],den2)

denb=poly2sym(dena)

numb=poly2sym(numr)

sr2=ilaplace(numb/denb)

sr3=((sr1-sr2)^2)

disp('**************INTEGRAL SQUARE ERROR OF THE REDUCED ORDER SYSTEM******************')

syms s x w t alpha

ise_reduced order = int(sr3,0,inf)

%******************CALCULATE THE VALUE OF IMPULSE RESPONSE ENERGY *************************

syms s t x w

den2n=poly2sym(den2)

num2n=poly2sym (numr)

sr4=ilaplace(num2n/den2n)

disp('**************IMPULSE RESPONSE ENERGY OF THE REDUCED ORDER SYSTEM******************')

syms s t w x alpha

ire_reduced order = int((sr4^2),0,inf)

## Appendix-B

MATLAB CODE DEVELOPED FOR ALGORITHM NO. 2

clc

clear

%--------------------------------------------

%PROJECT NAME

%--------------------------------------------

disp('PROJECT NAME : REDUCED ORDER MODELLING  USING :-')

disp(' ----> EIGEN SPECTRUM ANALYSIS & CAUER SECOND FORM')

%--------------------------------------------

%ENTER ORDER OF DENOMINATOR OF HIGH ORDERED SYSTEM (HOS)

%--------------------------------------------

nd=input('\nEnter order of denominator polynomial\n=')

%--------------------------------------------

%ENTER COEFFICIENT OF DENOMINATOR OF HOS

%--------------------------------------------

d=input('Enter coefficents of denominator polynomial\n=')

%--------------------------------------------

%ENTER ORDER OF LOWER ORDERED  SYSTEM (LOS)

%--------------------------------------------

p1=input('Enter required reduced order system\n=')

%--------------------------------------------

%ENTER ORDER OF NUMERATOR  OF HOS

%--------------------------------------------

nn=input('Enter order of numerator polynomial\n=')

%--------------------------------------------

%ENTER COEFFICIENT OF NUMERATOR OF HOS

%--------------------------------------------

d1=input('Enter coefficents of numerator polynomial\n=')

if nd==p1

disp('******* HOS AND LOS IS *******')

tf(d1,d)

step(d1,d,'g')

else

%--------------------------------------------

%ROOTS OF DENOMINATOR POLYNOMINAL OF HOS

%--------------------------------------------

rr=roots(d);

rr=real(rr);

%--------------------------------------------

%QUANTIFICATION OF POLE CENTROID & STIFFNESS OF HOS

%--------------------------------------------

centroid=mean(rr);

stiffness=rr(nd)/rr(1);

q=((p1-2)*(p1-1))/2;

k=[stiffness*(p1-1)+1 q;(1-stiffness) (1-p1)];

b=[centroid*2;0];

xx=inv(k)*b;

clear c

for i=1:p1

c(i)=xx(1)-(i-1)*xx(2);

end

%--------------------------------------------

% OBTAINED DENOMINATOR POLYNOMINAL OF LOS

%--------------------------------------------

pp=poly(c);

den=pp;

for i=1:(p1+1)

s(i)=den((p1+1)-i+1);

end

for i=1:(nd+1)

p(i)=d((nd+1)-i+1);

end

g(1,:)=p;

hhh=1;

clear q

t=s;

%--------------------------------------------

% OBTAINED HOS

%--------------------------------------------

disp('******* GIVEN HIGHER ORDERED SYSTEM (HOS) IS *******')

sys1=tf(d1,d)

t1=0:.001:10;

[y1,x1,t1]=step(d1,d,t1);

a1=max(y1);

%--------------------------------------------

% OBTAINED ZEROS, POLES & GAIN OF HOS

%--------------------------------------------

disp('******* ZEROS, POLES & GAIN OF HOS ARE *********')

[ZHOS,PHOS,KHOS]=tf2zp(d1,d)

%--------------------------------------------

% FORMING OF ROUTH ARRAY

%--------------------------------------------

for i=1:(nd)

q(i)=d1((nd)-i+1);

end

q=[q 0];

g(2,:)=q;

while hhh<nd

ww=hhh+1;

for i=1:((nd+1)-(hhh+1))

r(i)=(g(ww,1)*g(hhh,(i+1))-g(hhh,1)*g(ww,(i+1)))/g(ww,1);

end

ff=hhh+1;

oo=0;

while oo<ff

r=[r 0 ];

oo=oo+1;

end

rr=hhh+2;

g(rr,:)=r;

clear r

hhh=hhh+1;

end

%--------------------------------------------

% OBTAINED THE CAUER QUOTIENT

%--------------------------------------------

for i=1:(p1+1);

x(i)=((g(i,1))/g(i+1,1));

end

%--------------------------------------------

% EVALUATATION OF CAUER SECOND FORM COEFFICIENT

%--------------------------------------------

for i=1:(p1+1)

t(i+1,1)=t(i,1)/x(i);

end

j=1;

for j=1:(p1-1)

for i=1:(p1-j)

t(i+1,j+1)=(t(i,j+1)-t(i+2,j))/x(i);

i=i+1;

end

j=j+1;

end

op=t(2,:);

den;

%--------------------------------------------

% NUMERATOR TERMS OF LOS

%--------------------------------------------

for i=1:(p1+1)

num(i)=op(((p1+1)-i)+1);

end

%--------------------------------------------

% OBTAINED LOS

%--------------------------------------------

disp('********** LOWER ORDERED SYSTEM IS ***********')

%--------------------------------------------

% T.F. OF LOS

%--------------------------------------------

sys2=tf(num,den)

%--------------------------------------------

% STEP TIME FOR LOS

%--------------------------------------------

t2=0:.001:10;

[y2,x2,t2]=step(num,den,t2);

%--------------------------------------------

% OBTAINED MAXIMUM VALUE OF OUTPUT RESPONSE OF LOS

%--------------------------------------------

a2=max(y2);

%--------------------------------------------

% OBTAINED ZEROS , POLES & GAIN OF THE LOS

%--------------------------------------------

disp('******* ZEROS, POLES & GAIN OF LOS ARE *********')

[ZLOS,PLOS,KLOS]=tf2zp(num,den)

end

%--------------------------------------------

% CALCULATATION OF TIME SPECIFICATION OF HOS

%--------------------------------------------

disp('******* TIME SPECIFICATION OF HOS IS ********')

if a1<.9

disp('SYSTEM IS OVER DAMPED')

elseif a1>=.9 &a1<=.98

i=1;

while y1(i)<=0.9;

i=i+1;

end;

%--------------------------------------------

% CALCULATATION OF RISE TIME

%--------------------------------------------

Rise_Time1=(i-1)*0.001

elseif a1>=.98 &a1<=1

i=1;

while y1(i)<=0.9;

i=i+1;

end;

%--------------------------------------------

% CALCULATATION OF RISE TIME

%--------------------------------------------

Rise_Time1=(i-1)*0.001

i=10001;

while y1(i)<1.02&y1(i)>.98;

i=i-1;

end;

%--------------------------------------------

% CALCULATATION OF SETTLING TIME

%--------------------------------------------

Settling_Time1=(i-1)*.001

else

i=1;

while y1(i)<=0.9;

i=i+1;

end;

%--------------------------------------------

% CALCULATATION OF RISE TIME

%--------------------------------------------

Rise_Time1=(i-1)*0.001

%--------------------------------------------

% OBTAINED MAXIMUM VALUE OF OUTPUT RESPONSE OF HOS

%--------------------------------------------

[ymax1,tp1]=max(y1);

%--------------------------------------------

% CALCULATATION OF PEAK TIME

%--------------------------------------------

Peak_Time1=(tp1-1)*0.001

%--------------------------------------------

% CALCULATATION OF MAXIMUM OVER SHOOT

%--------------------------------------------

Max_Overshoot1=ymax1-1

i=10001;

while y1(i)<1.02&y1(i)>.98;

i=i-1;

end;

%--------------------------------------------

% CALCULATATION OF SETTLING TIME

%--------------------------------------------

Settling_Time1=(i-1)*.001

end;

%--------------------------------------------

% CALCULATATION OF TIME SPECIFICATION OF lOS

%--------------------------------------------

disp('******** TIME SPECIFICATION OF LOS IS **********')

if a2<.9

disp('SYSTEM IS OVER DAMPED')

elseif a2>=.9 &a2<=.98

i=1;

while y2(i)<=0.9;

i=i+1;

end;

%--------------------------------------------

% CALCULATATION OF RISE TIME

%--------------------------------------------

Rise_Time2=(i-1)*0.001

elseif a2>=.98 &a2<=1

i=1;

while y2(i)<=0.9;

i=i+1;

end;

%--------------------------------------------

% CALCULATATION OF RISE TIME

%--------------------------------------------

rise_time2=(i-1)*0.001

i=10001;

while y2(i)<1.02&y2(i)>.98;

i=i-1;

end;

%--------------------------------------------

% CALCULATATION OF SETTLING TIME

%--------------------------------------------

Settling_Time2=(i-1)*.001

else

i=1;

while y2(i)<=0.9;

i=i+1;

end;

%--------------------------------------------

% CALCULATATION OF RISE TIME

%--------------------------------------------

Rise_Time2=(i-1)*0.001

%--------------------------------------------

% OBTAINED MAXIMUM VALUE OF OUTPUT RESPONSE OF LOS

%--------------------------------------------

[ymax2,tp2]=max(y2);

%--------------------------------------------

% CALCULATATION OF PEAK TIME

%--------------------------------------------

Peak_Time2=(tp2-1)*0.001

%--------------------------------------------

% CALCULATATION OF MAXIMUM OVER SHOOT

%--------------------------------------------

Max_Overshoot2=ymax2-1

i=10001;

while y2(i)<1.02&y2(i)>.98;

i=i-1;

end;

%--------------------------------------------

% CALCULATATION OF SETTLING TIME

%--------------------------------------------

Settling_Time2=(i-1)*.001

end;

%--------------------------------------------

% CALCULATION OF INVERSE LAPLACE OF HOS

%--------------------------------------------

syms s t w x y

dnew=conv([1 0 ],d);

num_orig=poly2sym(d1);

den_orig=poly2sym(dnew);

y_orig=ilaplace(num_orig/den_orig);

%--------------------------------------------

% CALCULATION OF INVERSE LAPLACE OF LOS

%--------------------------------------------

dennew=conv([1 0 ],den);

num_red=poly2sym(num);

den_red=poly2sym(dennew);

y_red=ilaplace(num_red/den_red);

%--------------------------------------------

% CALCULATATION OF INTEGRAL SQUARE ERROR

%--------------------------------------------

yfinal=y_orig-y_red;

y_final=yfinal^2;

syms x x1 alpha u t;

disp('******* INTEGRAL SQUARE ERROR *********')

ISE_ERROR=int(y_final,0,inf)

%--------------------------------------------

% CALCULATATION OF IMPULSE RESPONSE ENERGY  OF HOS

%--------------------------------------------

den21_orig=poly2sym(d);

d1_orig=poly2sym(d1);

y21_orig=ilaplace(d1_orig/den21_orig);

y21_final=y21_orig^2;

disp('**** IMPULSE RESPONSE ENERGY  OF HOS ********')

IRE_ERROR=int(y21_final,0,inf)

%--------------------------------------------

% CALCULATATION OF IMPULSE RESPONSE ENERGY  OF LOS

%--------------------------------------------

den11_red=poly2sym(den);

y11_red=ilaplace(num_red/den11_red);

y11_final=y11_red^2;

disp('***** IMPULSE RESPONSE ENERGY  OF LOS ********')

IRE_ERROR=int(y11_final,0,inf)

%--------------------------------------------

% GRAPHICAL REPRESENTATION OF HOS & LOS

%--------------------------------------------

ltiview({'step';'pzmap'},sys1,'r',sys2,'b')

disp('*** PROJECT WAS COMPLETED UNDER THE GUIDANCE OF---------> Mrs. RAJNI JINDAL*** ')

disp('**************** THANK YOU ***********************')

## Appendix-C

SIMULINK MODELS FOR CALCULATION OF ISE & IRE

- Simulink model for calculation of ISE

- Simulink model for calculation of IRE