- ROBOTIC CONTROL BY NEURO-FUZZY APPROACH

- MAJOR THESIS SUBMITTED IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF THE DEGREE OF

- MASTER OF ENGINEERING

- IN

- CONTROL & INSTRUMENTATION

- SUBMITTED BY

EMANI.VENKATA REDDY

(Roll No.04/C&I/06)

UNIVERSITY ROLL NO.10214

- UNDER THE ESTEEMED GUIDANCE

- OF

- Mr. RAM BHAGAT

(LECTURER)

DEPARTMENT OF ELECTRICAL ENGINEERING                                                        DELHI COLLEGE OF ENGINEERING                                                        UNIVERSITY OF DELHI                                                                                            2006-2008

CERTIFICATE

- This is to certify that Major thesis titled “ROBOTIC CONTROL BY NEURO-FUZZY APPROACH” submitted by Mr. Emani.Venkata Reddy in partial fulfilment for the degree of Master of Engineering (Control & Instrumentation) of the Electrical Engineering Department, Delhi college of Engineering, Delhi – 110042 is a bonafide record of work, he has carried out under my guidance and supervision.

Mr.RAM BHAGAT

(Lecturer)

Electrical Engineering Department,

Delhi College of Engineering, Delhi

ACKNOWLEDGEMENT

It is with a great sense of pleasure that I acknowledge the help and guidance we have received from numerous people during the course of my project. I would like to extend my sincere gratitude and sincere thanks to my beloved guide Mr. RAM BHAGAT (Lecturer, Electrical Engineering Department, Delhi College of Engineering, Delhi) for his assistance and invaluable guidance towards the progress of this thesis.

I am very thankful to Prof. PARMOD KUMAR, Head of the Electrical Engineering Department, for providing valuable comments and supporting my effort.

I thank all the teaching and non teaching staff members of the department who have contributed directly or indirectly in successful completion of my thesis work. I also avail this opportunity to thank all my friends for their continuous support and encouragement.

Finally, I would like to say that I am indebted to my parents for everything that they have given to me. I thank them for the sacrifices they made so that I could grow up in learning environment. They have always stood by me in everything I have done, providing constant support, encouragement and love.

.

Mr. EMANI. VENKATA REDDY

M.E (C& I)

College Roll No: 04/C&I/06

ABSTRACT

The problem of manipulator control is highly complex problem of controlling a system which is multi-input, multi-output, and non-linear and time variant. A number of different approaches presently followed for the control of manipulator vary from PID to very complex, intelligent, self-learning control algorithms.

This report presents a comparative study of simulated performance of some conventional controllers, like the simple PID, Computed torque control, Feed forward inverse dynamic control and critically damped inverse dynamic control and some Intelligent controllers, like Fuzzy control, Neural control, and Neuro-Fuzzy control. IAE is used for comparison as performance index.

The study concludes that the critically damped inverse dynamics controller in general performs better then rest of conventional controllers. When the unmodeled term is added to the model, PID and Feed forward inverse dynamic control perform badly. Computed torque control and critically damped inverse dynamics control performance also affected but they do well. A Neuro-Fuzzy controller combines the advantage of neural networks (learning adaptability) with the advantage of fuzzy logic (use of expert knowledge) to achieve the goal of robust adaptive control of robot dynamics, performs better in intelligent controllers and also shows that intelligent controllers are better even when unmodeled terms are added to the model.

|  |  | Certificate | i |
| --- | --- | --- | --- |
|  |  | Acknowledgement | ii |
|  |  | Abstract | iii |
|  |  | Contents | iv |
|  |  | List of Figures | vi |
| Chapter 1 |  | Introduction | 1 |
|  | 1.1 | Introduction To Robot Control | 1 |
|  | 1.2 | Literature Review on Conventional Controllers | 3 |
|  | 1.3 | Literature Review On Fuzzy Control | 3 |
|  | 1.4 | Literature Review On Neural Control | 4 |
|  | 1.5 | Literature Review On Neuro-Fuzzy Control | 4 |
|  | 1.6 | Organization of the Dissertation | 5 |
| Chapter 2 |  | Dynamics Of Robotic Manipulator | 6 |
|  | 2.1 | Introduction | 6 |
|  | 2.2 | Puma Robot | 6 |
|  | 2.3 | Dynamics Of Puma560 Robot | 7 |
|  | 2.4 | Actuator For The Robot | 9 |
| Chapter 3 |  | Conventional Controllers | 12 |
|  | 3.1 | Introduction | 12 |
|  | 3.2 | PID control | 13 |
|  | 3.3 | Feed forward inverse dynamics control | 13 |
|  | 3.4 | Computed torque control | 14 |
|  | 3.5 | Critically damped inverse dynamics control | 15 |
| Chapter 4 |  | Fuzzy Control | 16 |
|  | 4.1 | Introduction | 16 |
|  | 4.2 | Historical Background | 16 |
|  | 4.3 | Structure Of Fuzzy Controller | 17 |
|  | 4.3.1 | Preprocessing | 17 |
|  | 4.3.2 | Fuzzification | 18 |
|  | 4.3.3 | Rule Base | 18 |
|  | 4.3.4 | Inference Engine | 19 |
|  | 4.3.5 | Defuzzification | 22 |
|  | 4.3.6 | Post Processing | 23 |
| Chapter 5 |  | Neural Control | 24 |
|  | 5.1 | Introduction | 24 |
|  | 5.2 | The Back-Propagation Algorithm | 25 |
|  | 5.3 | Neural Network Based Control | 29 |
| Chapter 6 |  | Neuro-Fuzzy Systems | 31 |
|  | 6.1 | Introduction | 31 |
|  | 6.2 | Adaptive Networks: Architectures And Learning Algorithms | 33 |
|  | 6.3 | ANFIS: Adaptive-Network-Based Fuzzy Inference System | 34 |
| Chapter 7 |  | Design and Simulation in Simulink/Matlab7.01 | 39 |
|  | 7.1 | Introduction | 39 |
|  | 7.2 | Puma560 Robot Model | 39 |
|  | 7.3 | Actuator Model | 42 |
|  | 7.4 | Design of PID Control | 43 |
|  | 7.5 | Design of Feed Forward Inverse Dynamics Control | 44 |
|  | 7.6 | Design of Computed Torque Control | 45 |
|  | 7.7 | Design of Critically Damped Inverse Dynamics Control | 46 |
|  | 7.8 | Design of Fuzzy PD+I Control | 47 |
|  | 7.9 | Design of Neural Control | 49 |
|  | 7.10 | Design of Neuro-Fuzzy Control | 51 |
| Chapter 8 |  | Results | 53 |
|  | 8.1 | Error Profiles of The Robot for Trajectory Control | 53 |
|  | 8.2 | Effect of Unmodeled Term on Performance | 56 |
| Chapter 9 |  | Conclusions | 60 |
|  |  | References |  |

LIST OF FIGURES

| Fig. 2.1 (a) puma 560 robot | 7 |
| --- | --- |
| Fig. 2.1 (b). Illustration of Puma 560 robot | 7 |
| Fig. 3.1 General structure of robot control system | 12 |
| Fig. 3.2 Feed forward inverse dynamics controller | 14 |
| Fig. 4.1 Blocks of a fuzzy controller | 17 |
| Fig. 4.2 General step response | 18 |
| Fig. 4.3 Graphical construction of the control signal in a fuzzy PD controller | 20 |
| Fig. 4.4 One input, one output rule base with non-singleton output sets | 21 |
| Fig. 5.1 Neural network architecture | 27 |
| Fig. 5.2 Neural controller | 29 |
| Fig. 5.3 Scheme for learning dynamic model | 30 |
| Fig. 6.1 A fuzzy system whose membership functions are adjusted by a neural network. | 31 |
| Fig. 6.2 A fuzzy system defined by a neural network | 32 |
| Fig. 6.3 Fuzzy neurons of a neural network | 32 |
| Fig. 6.4 A fuzzy system with neural network rule base | 33 |
| Fig. 6.5(a) fuzzy reasoning ;(b) equivalent ANFIS | 34 |
| Fig. 6.6 (a) fuzzy reasoning ;(b) equivalent ANFIS | 37 |
| Fig. 6.7 (a) 2-input ANFIS with 9 rules;(b) corresponding fuzzy subspaces | 38 |
| Fig. 7.1 PUMA560 simulink model | 39 |
| Fig. 7.2 dynamics matrices | 40 |
| Fig. 7.3 gravity load | 41 |
| Fig. 7.4 Matrix multiplications | 42 |
| Fig. 7.5 Actuator simulink model | 42 |
| Fig. 7.6 Total system with PID controller | 44 |
| Fig. 7.7 Total system with FFID controller | 44 |
| Fig. 7.8 Robot Response with PID control and FFID control | 45 |
| Fig. 7.9 Input block for CTC method | 45 |
| Fig. 7.10 Input block for CDID method | 46 |
| Fig. 7.11 Robot Response with CTC control and CDID control | 46 |
| Fig. 7.12 Fuzzy controls with gains | 47 |
| Fig. 7.13 Total system with fuzzy controller | 48 |
| Fig. 7.14 Fuzzy controller | 48 |
| Fig. 7.15 Total robot system with neural controller | 49 |
| Fig. 7.16 Neural Network1 block | 50 |
| Fig. 7.17 Robot response with Fuzzy control and Neural control | 50 |
| Fig. 7.18 Total robot system with ANFIS controller | 51 |
| Fig. 7.19 ANFIS controller | 52 |
| Fig.7.20 Robot Response with Neuro-Fuzzy control and membership functions after training | 52 |
| Fig.8.1.1 Position of robot with PID controller | 53 |
| Fig.8.1.2 Position of robot with CTC controller | 53 |
| Fig.8.1.3 Position of robot with FUZZY controller | 54 |
| Fig.8.1.4 Position of robot with ANFIS controller | 54 |
| Fig. 8.2.1-8.2.4 Error profiles of conventional control strategies without unmodeled term at each joint | 55 |
| Fig. 8.2.5-8.2.8 Error profiles of intelligent control strategies without unmodeled term at each joint | 56 |
| Fig. 8.2.9-8.2.18.Error profiles of PID, FFID, and Neural controllers with unmodeled term at each joint after network is trained to approximate inverse dynamics of robot with unmodeled term | 57-58 |